Name: ___________________________________________

Quiz #2 (Lectures 7-10)

1. What is the difference between the Maximum Contaminant Level (MCL) and Maximum Contaminant Level Goal (MCLG) in USEPA drinking water standards? Why aren’t these the same?

2. A 2-m x 2-m solar collector is used to heat water flowing into the device at 10 deg. C at a rate of 0.2 m3/s. The solar collector receives 500 W/m2 of incident solar radiation, of which 50% is transferred to the water. What is the temperature of the water flowing out of the collector?

3. You are reviewing the design for a set of four rapid mix basins operating in parallel (i.e., ¼ of discharge through each basin) will treat a total of 20,000 m3/d. The water has a viscosity of 1.2×10-3 kg/(m*s).

- For a desired contact time of 20-s in the rapid mix basin, what is the volume required for each of the four basins?

- The current design calls for each basin to be mixed by a 10 hp motor (i.e., the power input to the motor is 10 hp).  What will be the velocity gradient (G) in the basin? Report your answer in units of s-1.

- Typical design parameters for a basin would specify two conditions are met:

600 < G < 1000

5*104 < G < 5*105

Does the proposed design meet these criteria?

- Based on your responses above, would you recommend a larger or smaller motor, and why? What would change, if anything, if you take efficiency into account?