Name: ___________________________________________

Quiz #3 (Lectures 11-14)

1. Water Softening

Water to be softened has the following chemistry (all expressed in CaCO3 equivalents):

- CO32- = 17 mg/L

- HCO3- = 142 mg/L

- Ca2+ = 138 mg/L

- Mg2+ = 99 mg/L

- Na+ = 16 mg/L

- SO42- = 168 mg/L

- Cl- = 16 mg/L

- CO2 = 28 mg/L

A. Draw the bar diagram for the given information.

B. Report the following quantities:

Carbonate Hardness: 		____________

Noncarbonate Hardness: 	____________

Total Hardness: 		____________

B. How much lime (i.e., pure CaO) is required for the softening?

C. How much soda ash (i.e., pure Na2CO3) is required for the softening?

3. A horizontal flow settling basin was designed to treat 50 m3/s of discharge. The basin is 8-m wide, 2-m deep, and 50-m long. The basin was initially designed to remove suspended sediment, but it is now being evaluated for the potential to remove microplastics (i.e., tiny plastic particles; a density of 1500 kg/m3). Assume water density is 997 kg/m3. NEED A WATER VISCOSITY HERE

A. Consider a microplastic bead with a diameter of 0.05-mm and density of 1500 kg/m3. What fraction of these microplastics would be removed in the basin as presently designed?

B. What is the smallest diameter of microplastic that would be completely removed in the basin as presently designed?

C. If you wanted to increase removal of microplastics (i.e., capture more and/or smaller particles), how could you adjust the design or operation of the basin?

D. Assuming your answer to part C isn’t possible, what is another strategy you could use to capture microplastics?